I.  MATERIALS:

| Material | Source | Part # |
| --- | --- | --- |
| cDNA synthesis |  |  |
| Total RNA | Ambion |  |
| T7-Random Hexamer 5’GAATTGTAATACGACTCACTATAGGGN63’ | Operon |  |
| Random Primers, 3 μg/μL, 180 μg from 2-cycle cDNA kit or Random Primers, 3 μg/μL, 300 μg | Affymetrix Invitrogen | 900432 48190-011 |
| dNTP’s (dATP, dCTP, dGTP, dTTP), 100 mM dUTP, 100 mM | Promega | U1240 U1191 |
| RNase-free Water from 2-cycle cDNA kit or Nuclease-free Water 10X50 mL | Affymetrix Ambion | 900432 9937 |
| Rnase Inhibitor from 2-cycle cDNA kit or RNaseOUT, 40 U/μL, 5,000 U | Affymetrix Invitrogen | 900432 10777-019 |
| MgCl2, 1 M from 2-cycle cDNA kit or MgCl2, 1M, 100 mL | Affymetrix Ambion | 900432 9530G |
| SuperScript II, 5X 1st Strand Rxn. Mix, 0.1 M DTT from 2-cycle cDNA kit or SuperScript II, 200 U/μL, 40,000 U 5X First Strand buffer and 0.1 M DTT included | Affymetrix Invitrogen | 900432 18064-071 |
| Klenow Fragment (3’->5’ exo-), 5 U/μL, 1000 U | NEB | M0212L |
| RNase H, 2 U/μL, from 2-cycle cDNA kit or RNase H, 2 U/μL | Affymetrix Invitrogen | 900432 18021-071 |
| Poly-A RNA Control Kit |  |  |
| GeneChip® Eukaryotic Poly-A RNA Control Kit | Affymetrix | 900433 |
| cRNA Synthesis |  |  |
| MEGAscript® T7 Kit | Ambion | 1334 |
| Fragmentation and Labeling |  |  |
| Uracil DNA Glycosylase (2 U/μL) | NEB | M0280S |
| APE1 (10 U/μL) | NEB | M0282S |
| NEBuffer 4, 10X | NEB | B7004S |
| RNA 6000 Nano LabChip Kit | Agilent | 5065-4476 |
| RNA 6000 Ladder | Ambion | 7152 |
| Terminal Transferase, recombinant (30 U/μL) 5X Buffer TdT buffer included | Promega | M1875 |
| 0.5 M EDTA from 1-cycle cDNA kit or 0.5 M EDTA | Affymetrix Invitrogen | 900431 15575-020 |
| Microcon columns YM-3 | Millipore | 42404 |
| DLR-1a, 5 mM | Affymetrix | 900430 |

| cDNA, cRNA clean-up |  |  |
| --- | --- | --- |
| GeneChip Sample Cleanup Module | Affymetrix | 900371 |
| Hybridization, Stain and Wash |  |  |
| Human Exon Arrays | Affymetrix | TBD |
| 3 nM Control Oligo B2 | Affymetrix | 900301 or 900454 or 900457 |
| 20X Eukaryotic Hybridization Control | Affymetrix | 900457 or 900454 |
| Acetylated Bovine Serum Albumin (BSA) solution (50 mg/mL) | Invitrogen | 15561-020 |
| Herring Sperm DNA | Promega | D1811 |
| R-Phycoerythrin Streptavidin | Molecular Probes | S-866 |
| Goat IgG, Reagent Grade | Sigma | I 5256 |
| Anti-streptavidin antibody (goat), biotinylated | Vector Laboratories | BA-0500 |
| 20X Hybridization Buffer* | Various vendors |  |
| 2X Stain Buffer** | Various vendors |  |
| Wash Buffer A** | Various vendors |  |
| Wash Buffer B** | Various vendors |  |
| Miscellaneous Reagents |  |  |
| Absolute ethanol | Gold Shield Chemical Co. | N/A |
| Gel-Shift Assay (Optional) |  |  |
| Novex XCell SureLockTM Mini-Cell | Invitrogen | E10001 |
| TBE Gel, 4-20% ,10 mm, 12 well | Invitrogen | EC62252 |
| 5X Sucrose Gel Loading Dye | Amresco | E-274 |
| 10X TBE Buffer | Cambrex | 50843 |
| SYBR® Gold | Invitrogen | S-11494 |
| 10 b.p. DNA ladder and 100 b.p. DNA ladder | Invitrogen | 10821-015 15628-019 |
| ImmunoPure NeutrAvidin | Pierce | 31000 |
| 1M Tris, pH 7.0 | Ambion | 9850G |
| PBS, pH 7.2 | Invitrogen | 20012-027 |
| Miscellaneous Supplies |  |  |
| 1.5 mL RNase-free Microfuge Tubes | Ambion | 12400 |
| 1.5 mL Non-stick RNase-free Microfuge Tubes | Ambion | 12450 |
| 0.2 mL MicroAmp reaction tubes (8 tubes/strip) | Applied Biosystems | N801-0580 |
| MicroAmp caps for 8 strip tubes | Applied Biosystems | N801-0535 |
| Pipette for 25 mL*** | VWR | 53283-710 |
| Pipet-aid*** | VWR | 53498-103 |

| Instruments | Manufacturer | Part # |
| --- | --- | --- |
| NanoDrop® ND-1000*** | Ambion | N/A |
| GeneChip® Hybridization Oven 640*** | Affymetrix | 8001318 |
| Eppendorf Centrifuge*** | Eppendorf | 5417C |
| GeneChip® Fluidics Station 450*** | Affymetrix | 00-0079 |
| GeneArray™ Scanner 3000 upgraded for 5 μm Arrays Autoloader (Optional) | Affymetrix | 00-0073 90-0351 |
| ABI GeneAmp PCR System 2400*** | Applied Biosystems | N/A |
| Bioanalyzer 2100 | Agilent | G2940CA |
| Heating block*** | VWR | 13259-030 |
| Pipette for 0.1 to 2 μL *** | Rainin | L-2 |
| Pipette for 2 to 20 μL *** | Rainin | L-20 |
| Pipette for 20 to 200 μL *** | Rainin | L-200 |
| Pipette for 100 to 1000 μL*** | Rainin | L-1000 |

*See the GeneChip® Expression Analysis Technical Manual, pages 2.2.5 and 2.2.6 for the materials to make this buffer.

** See the GeneChip® Expression Analysis Technical Manual, pages 2.3.5 and 2.3.6 for the materials to make this buffer.

***Or equivalent instrument.

II.  PROCEDURE:

Preparation of total RNA with Poly-A RNA controls

A.  Poly-A RNA controls dilution

- The poly-A RNA controls are provided as a concentrated stock of 4 different transcripts at staggered concentrations.  Dilution buffer is provided with the kit to prepare the appropriate dilution based on Table 1 below.   Use the Non-stick RNase-free microfuge tubes to do the dilutions.

Table 1: Poly-A RNA control stock serial dilutions

| Starting ng of total RNA | Serial dilutions |  |  | Volume into 2.5 μg/μL T7-(N)6 Primer |
| --- | --- | --- | --- | --- |
|  | First | Second | Third |  |
| 100 ng | 1:20 | 1:50 | 1:50 | 2 μL |

- Add 2 μL of Poly-A RNA controls stock to 38 μL of Poly-A Control Dil Buffer to    make the First Dilution (1:20).

- Mix and spin down to collect at the bottom of tube.

- Take 2 μL of the First Dilution and add to 98 μL of Poly-A Control Dil Buffer to make the Second Dilution (1:50).

- Mix and spin down to collect at the bottom of tube.

- Take 2 μL of the Second Dilution and add to 98 μL of Poly-A Control Dil Buffer to make the Third Dilution (1:50).

- Mix and spin down to collect at the bottom of tube. Use the Third Dilution to prepare the solution described below.

B.  Preparation of T7-(N)6 Primer-poly-A RNA Controls

1.  Prepare a fresh 250 ng/μL T7-(N)6 Primer dilution from a 2.5 μg/μL stock. The diluted Poly-A RNA controls should be added to the concentrated T7-(N)6 Primer using a non-stick RNase-free microfuge tube as follows:

Table 2: Primer/Poly-A RNA controls

| Component | Volume |
| --- | --- |
| 2.5 μg/μL T7-(N)6 Primer | 2 μL |
| Diluted Poly-A RNA controls (3rd dilution) | 2 μL |
| RNase-free Water | 16 μL |
| Total volume | 20 μL |

2. Flick-mix the solution, spin down, and place on ice.

- Preparation of total RNA/Primer/Poly-A RNA controls

  - Mix total RNA, diluted T7-(N)6 Primer/Poly-A RNA controls solution as listed in the table below.

Table 3: Total RNA-Primer/Poly-A RNA controls

| Component | Volume in 1 Rxn. |
| --- | --- |
| Total RNA, 100 ng | variable |
| Diluted T7-(N)6 Primer/Poly-A RNA controls solution | 2 μL |
| RNase-free water | up to 5 μL |
| Total volume | 5 μL |

2.  Flick-mix and spin down the tube. Place on ice for use in the section below.

cDNA and cRNA synthesis

A.   First-Cycle, First Strand cDNA Synthesis

      - Incubate tube for 5 minutes at 70C, then cool the sample for at least 2 minutes at 4C, and spin down.

      - Prepare the 1st strand mix as shown in Table 4.

Table 4: First strand cDNA synthesis

| #### Component | Vol. in 1 Rxn |
| --- | --- |
| 5X 1st Strand Buffer | 2 μL |
| DTT, 0.1 M | 1 μL |
| dNTP*, 10 mM | 0.5 μL |
| RNaseOUT, 40 U/μL | 0.5 μL |
| SuperScript II, 200 U/μL | 1 μL |
| Total Volume | 5 μL |

*Mix 5 μL of 100mM dATP, 5 μL of 100mM dCTP , 5 μL of 100 mM dGTP,

and 5 μL of 100 mM dTTP with 30 μL of  RNase-free water.

Note: It is recommended to aliquot each of the dNTP’s stock solutions, as well as the working solution, to prevent degradation due to many freeze-thaw cycles.

      - Add 5 μL of the 1st strand mix to the tube containing the Total RNA/Primer/poly-A RNA controls, flick-mix, and spin-down. (The total reaction volume is 10 μL).

      - Incubate the reaction at 25C for 10 min., at 42C for 1 hour, and at 70C for 10 min. Then, cool the reaction to 4C for at least 2 minutes.  Continue to the 2nd  strand cDNA synthesis.

Note: Keeping the reaction at 4C longer than 10 min. may result in decreased cRNA yields.

- First-Cycle, Second Strand cDNA Synthesis

  - Prepare a 2nd strand mix as described in the table below.

Table 5: Second strand cDNA synthesis

| #### Component | Volume in 1 Rxn |
| --- | --- |
| MgCl2* 17.5 mM | 4.0 μL |
| dNTP, 10 mM | 0.4 μL |
| Klenow 3’5’ exo- , 5 U/ μL | 2.5 μL |
| RNase H, 2 U/ μL | 0.2 μL |
| RNase-free Water | 2.9 μL |
| Total Mix Volume | 10.0 μL |

* Make a fresh dilution of the MgCl2 each time.  Mix 2 μL of 1M  MgCl2

with 112 μL of RNase-free water.

  - Add 10 μL of the 2nd strand mix to the reaction tube from the 1st strand synthesis (total reaction volume is 20 μL), flick-mix the solution, and spin down.

  - Incubate for 50 min at 37°C, then 10 minutes at 75°C, and cool the sample at least 2 minutes at 4°C.

Note: Keeping the reaction at 4C longer than 10 min. may result in decreased cRNA yields.

- First-Cycle, cRNA Synthesis and Cleanup

  - In a separate tube, assemble the IVT mix at room temperature as listed in Table 6.

Table 6: cRNA synthesis with the MEGAscript T7 kit

| #### Component | Volume in 1 Rxn |
| --- | --- |
| 10X Reaction Buffer | 5.0 μL |
| ATP Solution | 5.0 μL |
| CTP Solution | 5.0 μL |
| UTP Solution | 5.0 μL |
| GTP Solution | 5.0 μL |
| Enzyme mix | 5.0 μL |
| Total Volume | 30.0 μL |

  - Transfer 30 µL of IVT mix to the 20 μL cDNA sample. Flick-mix the solution and briefly spin in a microfuge.

  - Incubate the reaction for 16 hours at 37°C.

  - Proceed to the clean-up procedure for cRNA using cRNA Cleanup Spin Columns from the GeneChip Sample Cleanup Module kit.

  - Store at -80C if not purifying the cRNA immediately.

  - If not already added, add 20 mL of Ethanol (100%) to the cRNA wash buffer supplied in the GeneChip Sample Cleanup Module kit.

  - Add 50 μL of RNase-free water to each IVT reaction.

  - Add 350 μL of cRNA Binding Buffer to each sample and vortex 3 seconds.

  - Add 250 μL of 100% ethanol to each reaction and flick-mix.

  - Apply the sample to the cRNA Column and centrifuge 15 seconds at >8,000 x g. Discard flow-through.

  - Transfer column to a new 2 mL collection tube and add 500 μL of cRNA wash buffer and spin as above. Discard flow-through.

  - Wash again with 500 μL of 80% (v/v) ethanol. Spin as above and discard flow-through.

  - Open column cap and spin at <25,000 x g (maximum speed) for 5 minutes.

  - Transfer column to a new 1.5 mL collection tube and add 13 μL of RNase-free water directly to the membrane. Spin at <25,000 x g (maximum speed) for 1 minute.

  - The eluted cRNA is ~11 μL.  Determine the cRNA yield by spectrophotometric UV measurement at 260nm, 280 nm and 320 nm:

Concentration of cRNA (μg/μL) = [A260  A320] x 0.04 x dilution factor

μg cRNA = eluate in μL x cRNA in μg/μL

Each tube should have an average of 20-60 μg of cRNA.

Note: this average yield range depends on the type of tissue used.

- Second-Cycle, First Strand cDNA Synthesis

Note: Perform all reactions in the Second-Cycle in triplicates

1.  Mix cRNA with the Random Primers in a strip tube, as listed in the table below.

Table 7: cRNA-Primer Mix

| #### Component | Volume in 1 Rxn |
| --- | --- |
| cRNA (5 μg) | variable |
| Random Primers (3μg/μL) | 1.0 μL |
| RNAse-free water | up to 8 μL |
| Total Volume | 8.0 μL |

  - Flick-mix, and spin down the tubes.

  - Incubate for 5 minutes at 70°C, 5 minutes at 25°C, and then cool the samples at 4°C for at least 2 minutes.

  - In a separate tube, prepare a Master mix as described in Table 8.

Table 8: First strand cDNA synthesis

| #### Component | Volume in 1 Rxn | Master mix for 3.5 Rxns |
| --- | --- | --- |
| 5X 1st Strand Buffer | 4.0 μL | 14.0 μL |
| DTT, 0.1 M | 2.0 μL | 7.0 μL |
| dNTP*, 10 mM (4dTTP:1dUTP) | 1.0 μL | 3.5 μL |
| RNaseOUT, 40 U/μL | 1.0 μL | 3.5 μL |
| SuperScript II, 200 U/μL | 4.0 μL | 14.0 μL |
| Total Volume | 12.0 μL | 42.0 μL |

*Mix 5 μL of 100 mM dATP, 5 μL of 100 mM dCTP , 5 μL of 100 mM dGTP, 4 μL of 100 mM dTTP,                       and 1 μL of 100 mM dUTP with 30 μL of  RNase-free Water.

- Transfer 12 µl of the 1st strand Master mix to the cRNA samples (the total reaction volume is 20 μL).  Mix thoroughly by gently flicking the tubes a few times.  Centrifuge briefly to collect the reactions at the bottom of the tube.

- Incubate the reactions at 25°C for 5 minutes, at 42°C for 1 hour, and 4°C for at least 2 min.

- Second-Cycle, Second Strand cDNA Synthesis and Cleanup

  - In a separate tube, assemble a Master mix as listed in the table below.

Table 9: Second strand cDNA synthesis

| #### Component | Volume in 1 Rxn | Master mix for 3.5 Rxns |
| --- | --- | --- |
| MgCl2* , 17.5 mM | 8.0 μL | 28.0 μL |
| dNTP, 10 mM (4 dTTP:1dUTP) | 0.6 μL | 2.1 μL |
| Klenow 3’-->5’ exo- , 5 U/ μL | 5.4 μL | 18.9 μL |
| RNase-free Water | 5.5 μL | 19.25 μL |
| RNAse H, 2U/ μL | 0.5 μL | 1.75 μL |
| Total Volume | 20 μL | 70.0 μL |

* Make a fresh dilution of the MgCl2 each time. Mix 2 μL of 1M  MgCl2 with 112 μL of                    RNase-free water.

  - Transfer 20 µl of the 2nd strand Master mix to the 1st strand reaction tubes (the total reaction volume is 40 μL).  Mix thoroughly by gently flicking the tubes a few times.  Centrifuge briefly to collect the reactions at the bottom of the tubes.

  - Incubate the reactions at 37C for 40 min., at 75C 10 min., and 4C for at least 2 min.

  - Proceed to the ds cDNA clean-up using the cDNA Cleanup Spin Columns from the GeneChip Sample Cleanup Module kit.

  - If not already added, add 24 mL of Ethanol (100%) to the cDNA wash buffer supplied in the GeneChip Sample Cleanup Module kit.

  - Add 60 μL of RNAse-free water to each sample, 370 μL of cDNA Binding Buffer, and vortex 3 seconds.

  - Apply 470 μL of sample to a cDNA spin column sitting in a 2 mL collection tube.

  - Spin at >8,000 x g for 1 minute. Discard flow-through.

  - Transfer column to new 2 mL collection tube and add 750 μL of cDNA Wash Buffer to column. Spin as in step 8, and discard flow-through.

  - Open cap of spin column, and spin at <25,000 x g for 5 minutes. Discard flow-through, and place column into a 1.5 mL collection tube.

  - Pipette 18 μL of the cDNA Elution Buffer directly to the column membrane and incubate at room temperature for 1 minute. Then, spin at <25,000 x g for 1 minute.

  - Repeat step 11.

  - The eluted ds cDNA is ~34 μL total.  Take 1 μL from each sample to determine the ds cDNA yield by spectrophotometric UV measurement at 260nm, 280 nm and 320 nm:

Concentration of ds cDNA (μg/μL) = [A260  A320] x 0.05 x dilution factor

μg ds cDNA = eluate in μL x ds cDNA in μg/μL

Each tube should have > 7 μg of ds DNA.

Fragmentation, Labeling, and Hybridization

A.  Fragmentation of ds cDNA

1.  Combine all three tubes of ds cDNA into one tube.

2.  Repeat step 13 from the previous section.

3.  Take 20 μg of ds DNA from the tube containing the pooled samples and split it into three tubes.

4.   Fragment each of the samples as listed in the table below:

Table 10: Fragmentation of ds cDNA

| Component | Volume/Amount in 1 Rxn. |
| --- | --- |
| ds cDNA | 6.67 μg |
| RNase free water | up to 32.2 μL |
| NEBuffer 4, 10X | 4.8 μL |
| Uracil DNA Glycosylase (2U/ μL) | 4.0 μL |
| APE1 (10U/ μL) | 7.0 μL |
| Total Volume | 48.0 μL |

      - Flick-mix, and spin down the tubes.

      - Incubate the reactions at 37C for 1 hour, 93C for 1 minute, and at 4C for at least 2 min.

      - Flick-mix, spin down the tubes, and remove 3 μL for fragmentation analysis using a Bioanalyzer.  Please see the Reagent Kit Guide that comes with the RNA 6000 Nano LabChip Kit.  The average peak size of the fragmented samples should be approximately 50 b.p.

      - If not labeling the samples immediately, store the fragmented ds cDNA

at -20C.

B.  Labeling of fragmented ds cDNA:

1.   Prepare the labeling reactions for the samples as listed in table 11:

Table 11: Labeling of fragmented ds cDNA

| Component | Volume in 1 Rxn. |
| --- | --- |
| Fragmented ds cDNA | 45.0 μL |
| 5x TdT Reaction buffer | 12.0 μL |
| rTdT, (30 U/uL) | 2.0 μL |
| DLR-1a, 5mM | 1.0 μL |
| Total Volume | 60.0 μL |

  - Flick-mix, and spin down the tubes.

  - Incubate the reactions at 37°C for 60 min, and cool at 4°C for at least 2 min.

  - Stop the reactions by adding 2 μL of 0.5 M EDTA (pH 8.0), flick-mix and spin down.

  - Remove 4 μL of each sample for Gel-shift analysis (optional), and combine samples into one tube (the total volume is 174 μL).

Note: If Gel-shift analysis were to be performed, please refer to the GeneChip® Expression Analysis Technical Manual, pages 3.1.6, 3.1.12, and 3.1.13.

- Concentration of three samples into one sample.

  - Pippette the sample into a YM-3 Microcon column already inserted into a 1.5 mL tube (tubes supplied with the Microcon columns).

  - Centrifuge at 14,000 x g for 40-50 min.  After 30 minutes, start measuring flow-through until the volume is close to 100 μL.

  - In a new tube, place column upside down and centrifuge at 1000 x g for 3 min.  The sample should be 68.7 μL.

Note: It is hard to get exactly 68.7 μL. If the volume is less than this, add RNase free water to bring the total volume up to 68.7 μL.

Note: Debris from the column membrane might result if spinning at maximum speed in step 3.

D.  Hybridization of Fragmented and labeled ds cDNA on the arrays (Std. arrays)

- Prepare the hybridization mix in a 1.5 mL RNase-free microfuge tube as shown in the table below:

Table 12: Hybridization Mix

| Component |  | Volume in 1 Rxn | Final Concentration or Amount |
| --- | --- | --- | --- |
| fragmented-labeled ds DNA |  | 68.7 μL | ~20 μg |
| Control Oligonucleotide B2 |  | 3.3 μL | 50 pM |
| 20X Eukaryotic Hybridization Controls (bioB, bioC, bioD, cre) |  | 10 μL | 1.5, 5, 25 and 100 pM, respectively |
| Herring Sperm DNA (10 mg/mL) |  | 2 μL | 0.1 mg/mL |
| Acetylated BSA (50 mg/mL) |  | 2 μL | 0.5 mg/mL |
| 2X Hybridization Buffer* |  | 100 μL | 1X |
| DMSO |  | 14 μL | 7% |
| Total Volume | 200.0 μL |  |  |

*Please refer to the GeneChip® Expression Analysis Technical Manual, page 2.2.6 for the preparation

of this buffer.

- Flick-mix, and centrifuge the tube.

- Heat the hybridization cocktail mix at 99°C for 5 min. Cool to 45°C for 5 minutes, and centrifuge at maximum speed for 1 minute.

- Label the Human Exon array for the sample that will be run.

- Inject ~200 μL of the specific sample into the array.

- Place array in 45°C hybridization oven, at 60 rpm, and incubate for about 16 hours.

E.  Array Wash, Stain, and Scan

- Use the fluidics protocol EuKGE-WS2v5_450 for wash and stain procedures as described in the GeneChip Expression Analysis Technical Manual (Section 2: Eukaryotic Sample and Array Processing).

- Scan the probe array according to the GeneChip Expression Analysis Technical Manual (Section 2: Eukaryotic Sample and Array Processing). Takes 30 min/chip.

- Generate .CHP files with Plier Algorithm.

    - DURATION OF ASSAY (3 days):

  - First Cycle: Synthesis of cDNA – ~3 1/2 hours – Day 1.

  - First Cycle: Synthesis of cRNA –16 hours – Start on Day 1 finish on Day 2.

  - Second Cycle: Synthesis of cDNA, fragmentation and labeling – 8 hours – Day 2.

  - Array Hybridization – 17 hours – Start on Day 3.

  - Array Wash, Stain and Scan – 3 hours – Day 3.